| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) 30th Meeting: Hangzhou, China, 23-27 October, 2006 | Document VCEG-AD08 Filename: VCEG-AD08.doc |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Yuri Vatis Institut für Informationsverarbeitung Leibniz Universität Hannover Appelstr. 9a 30167 Hannover Germany Joern Ostermann Institut für Informationsverarbeitung Leibniz Universität Hannover Appelstr. 9a 30167 Hannover Germany | Tel: Fax: Email: Tel: Fax: Email: | +49 511 762 5322 +49 511 762 5333 vatis@tnt.uni-hannover.de +49 511 762 5303 +49 511 762 5333 ostermann@tnt.uni-hannover.de |
| Title: | Prediction of P- and B-Frames Using a Two-dimensional Non-separable Adaptive Wiener Interpolation Filter for H.264/AVC |  |  |
| Purpose: | Proposal |  |  |

_____________________________

## Abstract

Standard video compression techniques apply motion-compensated prediction combined with transform coding of the prediction error. In the context of prediction with fractional-pel motion vector resolution it was shown, that aliasing components contained in an image signal are limiting the prediction efficiency obtained by motion compensation. In order to consider aliasing, quantisation and motion estimation errors, camera noise, etc., we analytically developed a two-dimensional (2D) non-separable interpolation filter, which is independently calculated for each frame by minimising the prediction error energy [1]. In contrast to [1] where the filter is applied only to P-Frames, we extended it here also to B-Frames, which becomes important if referenced B-Frames are applied. Thus, a coding gain of up to 0.4 dB Y-PSNR is achieved compared to [1] or up to 0.8 dB compared to the standard H.264/AVC High Profile.

## State of the Art

In order to reduce the bit rate of video signals, the ISO and ITU coding standards apply hybrid video coding with motion-compensated prediction combined with transform coding of the prediction error. In the first step the motion-compensated prediction is performed. The temporal redundancy, i.e. the correlation between consecutive images is exploited for the prediction of the current image from already transmitted images. In a second step, the residual error is transform coded, thus the spatial redundancy is reduced.

In order to perform the motion-compensated prediction, the current image of a sequence is split into blocks. For each block a displacement vector d→i is estimated and transmitted that refers to the corresponding position in a reference image. The displacement vectors have fractional-pel resolution. The standard H.264/AVC [3] is based on 1/4-pel displacement vector resolution. Displacement vectors with fractional resolution may refer to positions in the reference image, which are located between the sampled positions. In order to estimate and compensate the fractional-pel (sub-pel) displacements, the reference image has to be interpolated on the sub-pel positions. H.264/AVC uses a time-invariant 6-tap Wiener interpolation filter. The interpolation process used in H.264/AVC is depicted in Fig. 1 and can be subdivided into two steps. At first, the half-pel positions aa,bb,cc,dd,ee,ff and gg,hh,ii,kk,ll,mm are calculated using a horizontal or vertical 6-tap Wiener filter, respectively. Using the same Wiener filter applied at sub-pel positions aa,bb,cc,dd,ee,ff, the sub-pel position j is computed. In the second step, the residual quarter-pel positions are obtained, using a bilinear filter, applied at already calculated half-pel positions and existing full-pel positions.

Fig. 1: Integer samples (shaded blocks with upper-case letters) and fractional sample positions (white blocks with lower-case letters) for quarter-pel sample interpolation.

Wedi [3] proposed an adaptive interpolation filter, which is independently estimated for every image. This approach enables to take into account the alteration of image signal properties, especially aliasing, on the basis of minimisation of the prediction error energy. The filter coefficients used for the calculation of the half-pel positions are estimated iteratively using a numerical approach. The quarter-pel positions are calculated using a bilinear filter. In order to guarantee the convergence of the approach, the displacement vectors, estimated during the first step using the standard filter set, are used in further iterations.

In [1], we presented a new strategy for analytical calculation of optimal filter coefficients for a 2D non-separable motion compensated interpolation. The approach was applied for P-Frames. The achieved coding gain was up to 1 dB for Baseline Profile or up to 0.8 dB for Main Profile (IBBP). The complexity of this approach is discussed in [2]. The software was declared as an official VCEG KTA-software. In this paper, we extended the software to B-Frames, thus the coding gain is nearly kept for High Profile (I B RB B P).

This paper is organised as follows: First, the non-separable adaptive filter is described in detail. In order to reduce the side information for the filter coefficients, the necessary symmetry assumptions are explained. Further, we describe the necessary steps in order to adapt the proposed algorithm to B-Frames. The paper closes with experimental results.

## Non-separable two-dimensional Adaptive Wiener Interpolation filter

In this section we present an algorithm for analytical calculation of the coefficients of the optimal interpolation filter as described in [1]. For every sub-pel position SP (a … o), see Fig. 1, the individual set of coefficients is analytically calculated, such that no bilinear interpolation is used. If the sub-pel position to be interpolated is located at a,b,c,d,h,l, a one-dimensional 6-tap filter is calculated, using the samples C1−C6 for the sub-pel positions a,b,c and A3−F3 for d,h,l, respectively. For each of the remaining sub-pel positions, a two-dimensional 6x6-tap filter is calculated. For all sub-pel positions, the filter coefficients are calculated in a way that the prediction error energy is minimised. Note, that in this proposal we limit the size of the filter to 6x6 and the displacement vector resolution to a quarter-pel, but other filter sizes and displacement vector resolutions are also conceivable with our approach.

In the following, we describe the calculation of the filter coefficients more precisely. Let us assume that h00SP,h01SP, … ,h54SP,h55SP are the 36 filter coefficients of a 6x6-tap 2D filter used for a particular sub-pel position SP. Then the value pSP (a … o) to be interpolated is computed by a two-dimensional convolution:

| pSP=i=16j=16Pi,j hi−1,j−1SP |  |
| --- | --- |

where Pi,j is an integer sample value (A1 … F6).

The calculation of coefficients and the motion compensation are performed in the following steps:

- Displacement vectors d→t=mvx,mvy are estimated for every image to be coded. For the purpose of interpolation, the standard interpolation filter of H.264/AVC is applied to every reference image.

- 2D filter coefficients hi,j are calculated for each sub-pel position SP independently by minimisation of the prediction error energy:

| eSP2=xySx,y−ij hi,jSP Px+i,y+j2 |  |
| --- | --- |

with x=x+mvx−FO, y=y+mvy−FO

where Sx,y is an original image, Px,y a previously decoded image, i, j are the filter indices, mvxand mvy are the estimated displacement vector components, FO- a so called Filter Offset centring the filter (FO=filtersize2−1, in case of a 6-tap filter FO=2) and -operator is the floor function, which maps the estimated displacement vector mv to the next full-pel position smaller than mv. This is a necessary step, since the previously decoded images contain information only at full-pel positions. Note, for the error minimisation, only the sub-pel positions are used, which were referred to by motion vectors. Thus, for each of the sub-pel positions a… o an individual set of equations is set up by computing the derivative of eSP2 with respect to the filter coefficient hi,jSP. The number of equations is equal to the number of filter coefficients used for the current sub-pel position SP.

| 0 =∂eSP2∂hk,lSP =∂∂hk,lSPxySx,y−ij hi,jSP Px+i,y+j2 =xySx,y−ij hi,jSP Px+i,y+j Px+k,y+l∀k,l∈0;5 |  |
| --- | --- |

For each sub-pel position e,f,g,i,j,k,m,n,o using a 6x6-tap 2D filter, a system of 36 equations with 36 unknowns has to be solved. For the remaining sub-pel positions requiring a 1D filter, systems of 6 equations have to be solved. This results in 360 filter coefficients (nine 2D filter sets with 36 coefficients each and six 1D filter sets with 6 coefficients per set).

- New displacement vectors are estimated. For the purpose of interpolation, the adaptive interpolation filter computed in step 2 is applied. This step enables reducing motion estimation errors, caused by aliasing, camera noise, etc. on the one hand and to treat the problem in the rate-distortion sense on the other hand.

The steps 2 and 3 can be repeated, until a particular quality improvement threshold is achieved. Since some of the displacement vectors are different after the 3. step, it is conceivable to estimate new filter coefficients, adapted to the new displacement vectors. However, this would result in a higher encoder complexity.

## Symmetric two-dimensional Filter

Since transmitting 360 filter coefficients may result in a high additional bit rate, the coding gain can be drastically reduced, especially for video sequences with small spatial resolution. In order to reduce the side information, we assume that statistical properties of an image signal are symmetric. The following was described in [1] and implemented in the KTA-Software.

Thus, the filter coefficients are assumed to be equal, in case the distance of the corresponding full-pel positions to the current sub-pel position are equal (the distance equality between the pixels in x- and y-direction is also assumed, i.e. if the image signal is interlaced, a scaling factor should be considered, etc.).

Let us denote hC1a as a filter coefficient used for computing the interpolated pixel at sub-pel position a at the integer position C1, depicted in Fig. 1. The remaining filter coefficients are derived in the same manner. Then, based on symmetry assumptions only 5 independent 1D or 2D filter sets, consisting of different numbers of coefficients are required. Thus, for the sub-pel positions a,c,d,l only one filter with 6 coefficients is estimated, since:

| hC1a=hA3d=hC6c=hF3lhC2a=hB3d=hC5c=hE3lhC3a=hC3d=hC4c=hD3lhC4a=hD3d=hC3c=hC3lhC5a=hE3d=hC2c=hB3lhC6a=hF3d=hC1c=hA3l |  |
| --- | --- |

The same assumptions, applied at sub-pel positions b and h result in 3 coefficients for these sub-pel positions:

| hC1b=hC6b=hA3h=hF3hhC2b=hC5b=hB3h=hE3hhC3b=hC4b=hC3h=hD3h |  |
| --- | --- |

In the same way, we get 21 filter coefficients for sub-pel positions e,g,m,o, 18 filter coefficients for sub-pel positions f,i,k,n and 6 filter coefficients for the sub-pel position j. In total, this reduces the number of filter coefficients from 360 to 54, exploiting the assumption, that statistical properties of an image signal are symmetric.

The filter coefficients have to be quantised and transmitted as side information e.g. using an intra/inter-prediction and entropy coding [5].

## Adaptive Interpolation Filter for B-Frames

In [1] we applied the developed algorithm to P-Frames only. Here, we extended the algorithm also to B-Frames, which becomes very important if referenced B-Frames are used. The main difference between P- and B-Frames is, that in B-Frames bidirectional mode (or direct mode) is used [6]. Thus, one block can be predicted by averaging of two blocks of two different pictures. Therefore, in order to keep equal influence of each macroblock to the calculation of the optimal filter and not to weight macroblocks coded in a bidirectional (direct) mode twice, the equation (3) has been adapted as follows:

| 0 =∂∂hk,lSPxySx,y−12ij hi,jSP PF,x+i,y+j−12ij hi,jSP PB,x+i,y+j2 =xySx,y−12ij hi,jSP PF,x+i,y+j+PB,x+i,y+j PF,x+k,y+l+PB,x+k,y+l∀k,l∈0;5 |  |
| --- | --- |

with PF - forward reference frame and PB- backward reference frame.

Since the bit-rates of B-Frames are in general 30 to 50% lower than these of P-Frames during the side information for filter coefficients is nearly constant for both, P- and B-Frames, it can happen that the coding gain for B-Frames at low bit-rates is less than the cost of the side information. In order to avoid that, we propose the following algorithm, applied to B-Frames only, which does not increase the encoder complexity by estimating the gain achieved by means of adaptive interpolation filter. Namely, after the first and second steps the H.264-standard bitstream as well as side information for the filter coefficients are available. In case a particular threshold in ratio between the number of bits for side information and the number of bits for the standard bitstream is achieved (in our software empirically set to 0.1) then the filter coefficients are quantized coarser (7 bits per magnitude). If side information costs are still higher than the threshold, no additional interpolation and no additional coding step are performed, which is signalled by one additional bit in the slice header.

## Experimental Results

In our experiments for evaluating the coding efficiency of the adaptive Wiener interpolation filter, we coded several sequences of different resolutions (from CIF to HDTV 1080p). All simulations were performed using the High profile of H.264/AVC (JM10.2). The used settings are listed in Table 1.

| Resolution | CIF ( HDTV) |
| --- | --- |
| ProfileIDC | 100 |
| QPISlice | 23, 27, 31, 35 |
| IntraPeriod | 0 |
| QPPSlice | 24, 28, 32, 36 |
| NumberBFrames | 3 |
| QPBSlice | 25, 29, 33, 37 |
| BReferencePictures | 1 |
| PyramidCoding | 1 |
| PyramidLevelQPEnable | 1 |
| SearchRange | 32 (64) |
| NumberReferenceFrames | 5 (3) |
| SymbolMode | 1 |
| AdaptiveRounding | 1 |
| Transform8x8 | 1 |

Table 1: Important settings for the encoding process.

The rate-distortion curves show that the new approach (called “KTA+B”) outperforms the standard H.264/AVC for all bit rates. Performance gains of up to 0.8 dB or up to 25% bit-rate savings compared with H.264/AVC are achieved. Compared to the KTA, the coding gain of up to 0.4 dB is achieved, basically at high bit-rates. In some cases the coding losses at low bit-rates are observed (e.g. for the sequence Waterfall). This can be recouped by the adaptive usage of the interpolation filter (“KTA+B enhanced”).

Figure 1: Flowergarden, CIF, 30Hz, 240 Frames, I B RB B P

Figure 2: Mobile & Calendar, CIF, 30Hz, 300 Frames, I B RB B P

Figure 3: Concrete, CIF, 30Hz, 400 Frames, I B RB B P

Figure 4: Waterfall, CIF, 30Hz, 240 Frames, I B RB B P

Figure 5: Raven, HDTV(720p), 60Hz, 40 Frames, I B RB B P

Figure 6: Crew, HDTV(720p), 60Hz, 20 Frames, I B RB B P

Figure 7: City, HDTV(720p), 60Hz, 100 Frames, I B RB B P

Figure 8: Walking Couple, HDTV(1080p), 25Hz, 100 Frames, I B RB B P

Figure 9: Vintage Car, HDTV(1080p), 25Hz, 100 Frames, I B RB B P

Figure 10: Sunflower, HDTV(1080p), 25Hz, 100 Frames, I B RB B P

Figure 11: Rush Hour, HDTV(1080p), 25Hz, 80 Frames, I B RB B P

## IPR Statement

The Leibniz Universität Hannover may have IPR connected to the proposed method and based on reciprocity the Leibniz Universität Hannover is prepared to grant a license on reasonable and non-discriminatory terms.

## References

- Y. Vatis, B. Edler, D. T. Nguyen, J. Ostermann, “Two-dimensional non-separable Adaptive Wiener Interpolation Filter for H.264/AVC”, ITU-T SGI 6/Q.6 Doc. VCEG-Z17, Busan, South Korea, April 2005.

- Y. Vatis, J. Ostermann, “Comparison of Complexity between Two-dimensional non-separable Adaptive Interpolation Filter and Standard Wiener Filter”, ITU-T SG16/Q.6 Doc. VCEG-AA11, Nice, France, October 2005.

- ISO/IEC 14496-10:2003, “Coding of Audiovisual Objects - Part 10: Advanced Video Coding'', 2003, also ITU-T Recommendation H.264 “Advanced video coding for generic audiovisual services”.

- T.Wedi,”Adaptive Interpolation Filter for Motion Compensated Hybrid Video Coding”, Proc. Picture Coding Symposium (PCS2001), Seoul, Korea, April 2001

- Y. Vatis, B. Edler, I. Wassermann, D.T.Nguyen, J. Ostermann, “Coding of Coefficients of two-dimensional Adaptive Wiener Interpolation Filter”, Visual Communications and Image Processing (VCIP 2005), Beijing, China, July 2005.

- M. Flierl and B. Girod, “Generalized B Pictures and the Draft H.264/AVC Video-Compression Standard”, IEEE transactions on circuits and systems for video technology, vol. 15, No. 1, January 2005.